Preprocessing Plan

1. Overview

This pipeline downloads, harmonises, preprocesses and validates four multimodal time-series datasets: PAMAP2, WISDM, EEGMMIDB and PTB-XL. This produces fixed shape float32 arrays for downstream self-supervised learning. All outputs follow a consistent [N, C, T] shape with accompanying metadata CSVs and manifest files.

2. HAR: PAMAP2 and WISDM

Channel Schema

A 6-channel shared schema is used across both datasets: acc_x, acc_y, acc_z, gyr_x, gyr_y, gyr_z. PAMAP2 wrist IMU is paired with WISDM watch accelerometer and gyroscope.

Sampling Rate and Cleaning

The target rate is 20 Hz. WISDM is native at 20 Hz. PAMAP2 is downsampled from 100 Hz to 20 Hz using a 4th-order Butterworth low-pass filter with a 10 Hz cutoff (the Nyquist frequency for 20 Hz) applied (Nagima Abdrasulova et al., 2025) to prevent aliasing. NaN values are forward-filled up to 10 samples (0.1s) to maintain temporal continuity. PAMAP2 class 0 transient rows are dropped as they carry no activity label. Per-subject z-score normalisation is applied per channel across both datasets to remove inter-subject amplitude differences.

Windowing and Labels

Two outputs are produced. The pretraining set uses 10-second windows with no overlap and no labels (shape [N, 6, 200]). 10-second windows are long enough to capture complete activity cycles such as full gait cycles. The supervised set uses 5-second windows with 50% overlap and a majority vote label (shape [N, 6, 100]). 50% overlap increases the supervised dataset size without creating fully duplicate windows. Majority vote is used over first-label or last-label assignment because activity transitions within a window are common at 5-second resolution. Windows where no single class exceeds 50% of timesteps or where votes tie are discarded to prevent label noise into the training set. Seven shared classes are used: walking, jogging, stairs (PAMAP2 IDs 12+13 merged), sitting, standing, cycling, rope jumping. Subject IDs and source filenames are preserved in metadata.

2b. mHealth (Bonus HAR Dataset)

mHealth is included as a third HAR source. Right wrist IMU data is extracted from columns 15-20 (acc_x, acc_y, acc_z, gyr_x, gyr_y, gyr_z), matching the same 6-channel schema as PAMAP2 and WISDM. The native 50 Hz sampling rate is resampled to 20 Hz using resample_poly(2, 5). Label 0 (null class) is dropped as dataset documentation defines it as the null class, consistent with PAMAP2 class 0 handling. Six activities are mapped to the unified schema: standing (L1), sitting (L2), walking (L4), stairs (L5), cycling (L9) and jogging (L10). All other activities are excluded as they are not present in the shared schema. Per-subject z-score normalisation is applied. The same windowing strategy as PAMAP2 and WISDM is used

3. EEG: EEGMMIDB

Only runs 4, 8, and 12 are used. The motor imagery runs provide a consistent left-fist (T1) vs right-fist (T2) setting. T0 rest windows are excluded. The native 160 Hz rate is retained as motor imagery signals contain relevant content up to ~40 Hz and resampling can reduce the temporal precision and introduce artefacts (Mne.tools, 2025). Fixed 4-second windows are extracted starting at each T1 or T2 onset, giving shape [N, 64, 640]. A 1-40 Hz band pass filter removes slow drift and high-frequency noise (Lyu, 2026), a 60 Hz notch filter for power line interference, and common average referencing is applied across all 64 channels which were all kept to have a comprehensive dataset of brain activity (Ludwig et al., 2009). Subject ID, run ID, event code, and onset timing are preserved in metadata.

4. ECG: PTB-XL

100 Hz is selected over 500 Hz as it is sufficient for all clinically relevant ECG features (Kwon et al., 2018). Output shape is [N, 12, 1000] covering all 12 standard leads at 10 seconds. The dataset-provided strat_fold column is used directly: folds 1–8 as training, fold 9 as validation, fold 10 as test. Patient-level assignment is guaranteed by the dataset authors, preventing leakage. A 0.5 Hz high-pass filter removes baseline wander (Luo et al., 2013), a 50 Hz notch filter removes European power line interference (Zhou and Zhang, 2013) and per-lead z-score normalisation is applied (Al-Zaiti et al., 2023). Patient ID, record ID, SCP codes, lead names and fold are preserved in metadata.

5. Storage, RAM, and Engineering Risks

Raw storage: ~8.5 GB total (PAMAP2: 2.9 GB, WISDM: 1.5 GB, EEGMMIDB: 3.4 GB, PTB-XL: 616 MB, mHealth: 72 MB). Processed storage: ~1.9 GB. Peak RAM: ~4 GB during EEG processing, handled by subject-by-subject chunking. PAMAP2 and mHealth preprocessing are also performed per-subject. Estimated runtime: ~50 minutes excluding downloads.

Key engineering risks:

- PAMAP2 NaN gaps: forward-fill capped at 0.1s; longer gaps result in window exclusion to avoid interpolating across sensor dropout events.

- EEG annotation parsing: only onset timestamps used, not event duration, to guarantee consistent 4-second windows across all 109 subjects.

- PTB-XL multi-label records: the SCP diagnostic superclass with the highest annotator likelihood is used as the primary label. All codes are preserved in metadata for downstream use.

- WISDM watch accelerometer and gyroscope files are merged by index alignment after sorting by timestamp. Only .txt raw files are used; .arff files are excluded.

References:

Al-Zaiti, S.S., Martin-Gill, C., Zègre-Hemsey, J.K., Bouzid, Z., Faramand, Z., Alrawashdeh, M.O., Gregg, R.E., Helman, S., Riek, N.T., Kraevsky-Phillips, K., Clermont, G., Akcakaya, M., Sereika, S.M., Van Dam, P., Smith, S.W., Birnbaum, Y., Saba, S., Sejdic, E. and Callaway, C.W. (2023). Machine learning for ECG diagnosis and risk stratification of occlusion myocardial infarction. Nature Medicine, [online] 29, pp.1–10. doi:https://doi.org/10.1038/s41591-023-02396-3.

Kwon, O., Jeong, J., Kim, H.B., Kwon, I.H., Park, S.Y., Kim, J.E. and Choi, Y. (2018). Electrocardiogram Sampling Frequency Range Acceptable for Heart Rate Variability Analysis. Healthcare Informatics Research, [online] 24(3), p.198. doi:https://doi.org/10.4258/hir.2018.24.3.198.

Ludwig, K.A., Miriani, R.M., Langhals, N.B., Joseph, M.D., Anderson, D.J. and Kipke, D.R. (2009). Using a Common Average Reference to Improve Cortical Neuron Recordings From Microelectrode Arrays. Journal of Neurophysiology, 101(3), pp.1679–1689. doi:https://doi.org/10.1152/jn.90989.2008.

Luo, Y., Hargraves, R.H., Belle, A., Bai, O., Qi, X., Ward, K.R., Pfaffenberger, M.P. and Najarian, K. (2013). A Hierarchical Method for Removal of Baseline Drift from Biomedical Signals: Application in ECG Analysis. The Scientific World Journal, [online] 2013, p.e896056. doi:https://doi.org/10.1155/2013/896056.

Lyu, R. (2026). Deep learning approaches for EEG-based healthcare applications: a comprehensive review. Frontiers in Human Neuroscience, 19. doi:https://doi.org/10.3389/fnhum.2025.1689073.

Mne.tools. (2025). Filtering and resampling data — MNE 1.11.0 documentation. [online] Available at: https://mne.tools/stable/auto_tutorials/preprocessing/30_filtering_resampling.html.

Nagima Abdrasulova, Aleksanyan, M., Kim, M.J., Ahn, J.M., Nagima Abdrasulova, Aleksanyan, M., Kim, M.J. and Ahn, J.M. (2025). Butterworth Filtering at 500 Hz Optimizes PPG-Based Heart Rate Variability Analysis for Wearable Devices: A Comparative Study. Sensors, [online] 25(22), pp.7091–7091. doi:https://doi.org/10.3390/s25227091.

Zhou, X. and Zhang, Y. (2013). A hybrid approach to the simultaneous eliminating of power-line interference and associated ringing artifacts in electrocardiograms. BioMedical engineering online, 12(1), pp.42–42. doi:https://doi.org/10.1186/1475-925x-12-42.